| Flag of Ship |  |
| --- | --- |
| IMO Number |  |
| Name of Ship |  |

|  | Questionnaire | Req. | YES | NO | NA | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Title 3 | Accommodation and recreational facilities |  |  |  |  |  |
| 1 | Accommodation (General requirements) |  |  |  |  |  |
| 1.1 | The headroom satisfies the minimum requirement of 203cm (Limited reduction may be applied if permitted by the competent authority) | A3.1.6a |  |  |  |  |
| 1.2 | The accommodation (including sleeping rooms, mess rooms and recreation rooms and alleyways) is adequately insulated to prevent condensation or overheating | A3.1.6b |  |  |  |  |
| 1.3 | The sleeping rooms (SRs) are located above the loadline and either amidships or aft (in ships other than passenger ships and special ships) | A3.1.6c |  |  |  |  |
| 1.4 | All SRs are located forward of the collision bulkhead (Not permitted) | A3.1.6c |  |  |  |  |
| 1.5 | There are any direct openings between cargo, machinery spaces, galleys, storerooms, drying rooms or communal sanitary areas to any of the sleeping rooms | A3.1.6e |  |  |  |  |
| 1.6 | The external bulkheads and part bulkheads that separates cargo, machinery spaces, galleys, store rooms, drying rooms and communal sanitary areas from sleeping rooms are constructed of steel / other approved substance and watertight and gas-tight | A3.1.6e |  |  |  |  |
| 1.7 | The materials are used to construct internal bulkheads, panelling and sheeting, floors and joining suitable for the purpose and conducive to ensuring a healthy environment | A3.1.6f |  |  |  |  |
| 1.8 | Proper lighting and sufficient drainage are provided in the accommodation areas | A3.1.6g |  |  |  |  |
| 1.9 | The accommodation, recreation and catering facilities meet the requirements of Reg. 4.3 and relevant parts of the convention on: | A3.1.6h R. 4.3 | - | - |  |  |
| a | health and safety protection and accident prevention |  |  |  |  |  |
| b | the prevention of exposure to hazardous levels of noise and vibration |  |  |  |  |  |
| c | the prevention of exposure to hazardous ambient factors and chemicals on board |  |  |  |  |  |
| d | the provision of acceptable occupational and on board living environmental conditions |  |  |  |  |  |
| 1.10 | Seafarers have access to space or spaces on open deck that are accessible when off duty | A3.1.14 |  |  |  |  |
| 1.11 | The ship regularly trading to mosquito-infested ports is appropriate devices fitted as set out in the national requirements (ref. DMLC Part I) | A3.1.16 |  |  |  |  |
| 1.12 | Records of frequent inspections by the master or designated officer of the seafarers’ accommodations (in accordance with the DMLC Part II, where applicable) are available | A3.1.18 |  |  |  |  |
| 2 | Noise and Vibration |  |  |  |  |  |
| 2.1 | Comply with Code of noise or revised code of Noise according to the date of contract of ship concerned | A3.1.4c |  |  |  |  |
| 3 | Ventilation and Air-condition |  |  |  |  |  |
| 3.1 | All sleeping rooms and mess rooms are adequately ventilated | A3.1.7a |  |  |  |  |
| 3.2 | The ship is equipped with air conditioning for the seafarer accommodation, for any separate radio room and for any centralized machinery control room | A3.1.7b |  |  |  |  |
| 3.3 | Sanitary spaces are ventilated to open air independently of other accommodation | A3.1.7c |  |  |  |  |
| 4 | Heating |  |  |  |  |  |
| 4.1 | An appropriate heating system of seafarer accommodation is provided | A3.1.7d |  |  |  |  |
| 5 | Sleeping Room (SR) |  |  |  |  |  |
| 5.1 | There are any direct openings between cargo, machinery spaces, galleys, storerooms, drying rooms or communal sanitary areas to any of the sleeping rooms (Not permitted) | A3.1.6e |  |  |  |  |
| 5.2 | All SRs are adequately ventilated | A3.1.7a |  |  |  |  |
| 5.3 | SRs are lit by natural light and provided with adequate artificial light in accordance with relevant national standards (Special arrangements may be set for passenger ships) (ref. DMLC Part I) | A3.1.8 |  |  |  |  |
| 5.4 | Individual SRs are provided for each seafarer (Definition of 'seafarer' to be found in DMLC Part I.) (Exemption may be applied to the ships of less than 3,000 GRT) (This requirement does not apply to passenger ships) | A3.1.9a |  |  |  |  |
| 5.5 | Separate SRs are provided for women and for men | A3.1.9b |  |  |  |  |
| 5.6 | SRs be of adequate size and be properly equipped to provide, comfort and to facilitate tidiness are provided | A3.1.9c |  |  |  |  |
| 5.7 | A separate sleeping berth is provided for each seafarer | A3.1.9d |  |  |  |  |
| 5.8 | The minimum inside size of a berth dimensions are at least 198 x 80 cm | A3.1.9e |  |  |  |  |
| 5.9 | The floor area in all single berth SRs satisfy the following minimum size requirements: | A3.1.9f | - | - |  |  |
| a | 4.5m² in ships of less than 3,000 GRT (A reduction in size may be applied if permitted by the competent authority :ref DMLC Part I) | A3.1.9f.i A3.1.9g |  |  |  |  |
| b | 5.5m² in ships of 3,000 GRT or over but less than 10,000 GRT | A3.1.9f.ii |  |  |  |  |
| c | 7m² in ships of 10,000 GRT or more | A3.1.9f.iii |  |  |  |  |
| d | On ships of less than 3,000 GRT other than passenger ships and special purpose ships: - the maximum occupancy of no more than 2 persons per SR met, and the floor area is not less than 7m² | A3.1.9h |  |  |  |  |
| 5.10 | On passenger ships and special purpose ships: - the floor area of SRs, which are occupied seafarers not performing the duties of ship's officers, are not less than: | A3.1.9i | - | - |  |  |
| a | 7.5m² - 2 persons | A3.1.9i.i |  |  |  |  |
| b | 11.5m² - 3 persons | A3.1.9i.ii |  |  |  |  |
| c | 14.5m² - 4 persons | A3.1.9i.iii |  |  |  |  |
| 5.11 | On special purpose ships: - the floor of SRs when occupied by more than 4 persons per room is not less than 3.6m² per person | A3.1.9j |  |  |  |  |
| 5.12 | On passenger ships and special purpose ships: - the floor area of SRs for junior officers are not less than 7.5m² when no private sitting room / day room is provided | A3.1.9l |  |  |  |  |
| 5.13 | On passenger ships and special purpose ships: - the floor area of SRs for senior officers are not less than 8.5m² when no private sitting room / day room is provided | A3.1.9l |  |  |  |  |
| 5.14 | On ships other than passenger ships and special purpose ships: - the floor area of SRs for ships' officers, where there is no private sitting room or day room provided, are not less than; | A3.1.9k | - | - |  |  |
| a | 7.5m² in ships of less than 3,000 GRT | A3.1.9k.i |  |  |  |  |
| b | 8.5m² in ships of 3,000 GRT or over but less than 10,000GRT | A3.1.9k.ii |  |  |  |  |
| c | 10m² in ships of 10,000GRT or over | A3.1.9k.iii |  |  |  |  |
| 5.15 | The master, the chief engineer and the chief navigating officer (including second engineer, if practicable) have an adjoining sitting room, day room or an equivalent additional space in addition to their SR (Exemption may be applied to the ships of less than 3,000 GRT) | A3.1.9m (B3.1.5.5) |  |  |  |  |
| 5.16 | In each SR provided to each seafarer, with a clothes locker (minimum 475 litters) and a drawer or equivalent space of not less than 56 litters capacity Combined units are permitted but shall be 500 litters minimum. All lockers are provided with a shelf and are able to be locked | A3.1.9n |  |  |  |  |
| 5.17 | Each SR is provided with a table or desk and comfortable seating accommodation as necessary | A3.1.9o |  |  |  |  |
| 5.18 | Each SR is provided a washbasin with hot and cold running fresh water; except in cases when such a washbasin is situated in a private bathroom (This does not apply to passenger ships) | A3.1.11d |  |  |  |  |
| 6 | Mess Room (MR) |  |  |  |  |  |
| 6.1 | MRs are adequately ventilated | A3.1.7a |  |  |  |  |
| 6.2 | MRs are lit by natural light and provided with adequate artificial light | A3.1.8 |  |  |  |  |
| 6.3 | MRs are located as close as possible to the galley and apart from SRs (Exemption may be applied to the ships of less than 3,000 GRT) | A3.1.10a |  |  |  |  |
| 6.4 | MRs are to be of adequate size and comfort and properly furnished and equipped, taking account of the number of seafarers likely to use them at any one time; provision shall be made for separate or common mess room facilities as appropriate | A3.1.10b |  |  |  |  |
| 7 | Recreational Facilities (RFs) |  |  |  |  |  |
| 7.1 | As recreational facilities (RFs) are amenities and services are provided for the seafarer on board ship | A3.1.17 |  |  |  |  |
| 8 | Sanitary Facilities |  |  |  |  |  |
| 8.1 | All toilets and other sanitary facilities (SFs) are functioning properly | A3.1.11a |  |  |  |  |
| 8.2 | Separate SFs are provided for men and for women | A3.1.11a |  |  |  |  |
| 8.3 | All seafarers have convenient access to SFs | A3.1.11a |  |  |  |  |
| 8.4 | There are SFs within easy access of the navigating bridge and the machinery space or near the engine room control center (Exemption may be applied to the ships of less than 3,000 GRT) | A3.1.11b |  |  |  |  |
| 8.5 | The following SFs are provided for at a convenient location for those seafarers who do not have personal SFs for every 6 persons or less, a minimum of: | A3.1.11c | - | - |  |  |
| a | 1 toilet |  |  |  |  |  |
| b | 1 wash basin |  |  |  |  |  |
| c | 1 tub or shower or both |  |  |  |  |  |
| 8.6 | On passenger ships, normally engaged on voyages of not more than 4 hours duration, are all the special arrangements or exemptions with regard to the SFs in accordance to that specified by the competent authority (ref. DMLC Part I) | A3.1.11e |  |  |  |  |
| 8.7 | Hot and cold running fresh water is available in all wash places | A3.1.11f |  |  |  |  |
| 9 | Laundry Facilities (LF) |  |  |  |  |  |
| 9.1 | Appropriately located and equipped laundry facilities are provided on board the ship | A3.1.13 |  |  |  |  |
| 10 | Hospital Accommodation (HA) |  |  |  |  |  |
| 10.1 | Separate hospital accommodation (HA) is provided on board for exclusively medical purposes (applies to ships carrying 15 or more seafarers and engaged in voyages of more than 3 days) (Exemption may be applied to ships engaged in coastal trade) | A3.1.12 |  |  |  |  |
| 10.2 | The HA is easy to access, comfortable and conducive to the occupants' receiving prompt and proper attention | A3.1.12 |  |  |  |  |
| 11 | Offices |  |  |  |  |  |
| 11.1 | Separate or a common ship's office are provided for use by deck and engine departments on the ship (Exemption may be applied to the ships of less than 3,000 GRT) | A3.1.15 |  |  |  |  |